Circuite VLSI - Proiect

Circuit secvențial de împărțire fără restaurare

numere pozitive

Autor:

Pârvan Andrei Leonard

Calculatoare anul 4

Grupa 4LF781

andrei.parvan@student.unitbv.ro

BRAȘOV 2022

Specificații:

- Schema bloc a circuitului:

- Descrierea porturilor:

- Algoritmul utilizat:

Algoritm de împărțire fără restaurare (numere pozitive)

- resetează P (n+1 biți)

- încarcă deîmpărțitul în A (n biți)

- încarcă împărțitorul în B (n biți)

- repeta de n ori

- dacă P este negativ (MSB=1) atunci

- deplasează cu o poziție stânga P (LSB P = MSB A)

- P <= P + B

altfel

- deplasează cu o poziție stânga P (LSB P = MSB A)

- P <= P + (-B)

- deplasează cu o poziție stânga A (LSB A = not MSB P)

- dacă P este negativ (MSB=1) atunci

- P <= P + B

- P conține REST

- A conține CÂT

- Constrângeri:

Sistemul se va modela ca un sistem riguros sincron. Semnalul ck va fi primit de către toate registrele din sistem.

Se vor evita circuitele de divizare a frecvenței semnalului de tact.

Se vor folosi, eventual, circuite de generare a semnalelor de activare (enable).

Descrierea sistemului se va face în întregime în VHDL. În descrierea de nivel înalt se vor instanția două componente:

- cale de date

- cale de control

Exemplu:

75 : 10 = 7 rest 5 				85 : 10 = 8 rest 5

75 = 0 0100 1011 				85 = 0 0101 0101

10 = 0 0000 1010 				10 = 0 0000 1010

-10 = 1 1111 0110 				-10 = 1 1111 0110

Reg. P 			Reg. A	 		Reg. P	 		Reg. A

-----------		--------- 		----------- 		---------

0 0000 0000 		0100 1011 		0 0000 0000 		0101 0101 - inițial

0 0000 0000 					0 0000 0000

1 1111 0110 					1 1111 0110

----------- 					-----------

1 1111 0110 		1001 0110 		1 1111 0110 		1010 1010 - iterația 1

1 1110 1101 					1 1110 1101

0 0000 1010 					0 0000 1010

-----------					 -----------

1 1111 0111		0010 1100 		1 1111 0111		0101 0100 - iterația 2

1 1110 1110 					1 1110 1110

0 0000 1010 					0 0000 1010

----------- 					-----------

1 1111 1000 		0101 1000 		1 1111 1000 		1010 1000 - iterația 3

1 1111 0000 					1 1111 0001

0 0000 1010 					0 0000 1010

----------- 					-----------

1 1111 1010 		1011 0000 		1 1111 1011 		0101 0000 - iterația 4

1 1111 0101 					1 1111 0110

0 0000 1010 					0 0000 1010

----------- 					-----------

1 1111 1111 		0110 0000 		0 0000 0000 		1010 0001 - iterația 5

1 1111 1110 					0 0000 0001

0 0000 1010 					1 1111 0110

----------- 					-----------

0 0000 1000 		1100 0001 		1 1111 0111 		0100 0010 - iterația 6

0 0001 0001 					1 1110 1110

1 1111 0110 					0 0000 1010

----------- 					-----------

0 0000 0111 		1000 0011 		1 1111 1000 		1000 0100 - iterația 7

0 0000 1111 					1 1111 0001

1 1111 0110 					0 0000 1010

----------- 					-----------

0 0000 0101		0000 0111 		1 1111 1011 		0000 1000 - iterația 8

1 1111 1011			         - final

0 0000 1010

-----------

0 0000 0101

0000 0101 		0000 0111 		    0000 0101 		0000 1000

--------- 		--------- 		--------- 		---------

REST = 5 		CÂT = 7 		REST = 5 		CÂT = 8

Reprezentare grafică a structurii de nivel înalt

Graficul de stari

Simulare circuit în ModelSIM

Fișier date.vhd

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

USE IEEE.std_logic_arith.ALL;

USE IEEE.std_logic_unsigned.ALL;

ENTITY date IS

PORT (

clk           : IN  std_logic;                        --clock

reset         : IN  std_logic;                        --reset

a             : IN  std_logic_vector(3 DOWNTO 0);     --operandul A

b             : IN  std_logic_vector(3 DOWNTO 0);     --operandul B

start             : IN std_logic;                     -- semnale pentru stari

initizlizare      : IN std_logic;                     -- --||--

pauza             : IN std_logic;                     -- --||--

validare_p        : IN std_logic;                     -- --||--

adunare_b         : IN std_logic;                     -- --||--

adunare_minus_b   : IN std_logic;                     -- --||--

deplasare_a       : IN std_logic;                     -- --||--

validare_finala_p : IN std_logic;                     -- --||--

adunare           : IN std_logic;                     -- --||--

confirmare        : IN std_logic;                     -- --||--

cat                 : OUT std_logic_vector(3 DOWNTO 0); --catul

rest                : OUT std_logic_vector(4 DOWNTO 0); --restul

numarator         : OUT std_logic; --necesar pentru Calea de control

pmsb              : OUT std_logic --necesar pentru Calea de control

);

END date;

ARCHITECTURE date OF date IS

SIGNAL reg_a      : std_logic_vector(3 DOWNTO 0);      -- a pe n biti

SIGNAL reg_b      : std_logic_vector(3 DOWNTO 0);      -- b pe n biti

SIGNAL reg_p      : std_logic_vector(4 DOWNTO 0);      -- p pe n+1 biti

SIGNAL count      : std_logic_vector(2 DOWNTO 0);      -- ce e asta

BEGIN

--reg A

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN                                                 -- la fiecare event de clock

IF  (reset = '1')     THEN reg_a <= (others => '0');                            -- daca reset e 1 atunci reg_a primeste 0?

ELSIF (initizlizare = '1')THEN reg_a <= a;                                      -- daca init activ reg_a primeste a

ELSIF (deplasare_a = '1') THEN reg_a <= reg_a(2 DOWNTO 0) & ( not reg_p(4));      -- daca deplasare a activ reg este shiftat la stanga cu un bit

END IF;

END IF;

END PROCESS;

--reg B

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN                           -- la fiecare event de clock

IF  (reset = '1')     THEN reg_b <= (others => '0');      -- reg_b primeste 0 la reset activ

ELSIF (initizlizare = '1')THEN reg_b <= b;                -- daca init activ reg_b primeste b

END IF;

END IF;

END PROCESS;

--reg P

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN                                                         -- la fiecare event de clock

IF  (reset = '1')       THEN reg_p <= (others => '0');                                  -- reset activ reg_p primeste 0

ELSIF (initizlizare = '1')  THEN reg_p <= (others => '0');                              -- initializarea reg_p primeste 0

ELSIF (adunare = '1')     THEN reg_p <= reg_p + reg_b;                                    -- cand adunare este activ se aduna reg_P cu reg_b

ELSIF (adunare_b = '1')     THEN reg_p <= (reg_p(3 DOWNTO 0) & reg_a(3)) + reg_b  ;     -- se aduna fie B fie (-B)

ELSIF (adunare_minus_b = '1') THEN reg_p <= (reg_p(3 DOWNTO 0) & reg_a(3)) - reg_b ;

END IF;

END IF;

END PROCESS;

--reg count

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN

IF  ( reset = '1' )     THEN count <= (others => '0');                                  -- initializare la reset sau init

ELSIF ( initizlizare = '1' )  THEN count <= (others => '0');

ELSIF ( validare_p = '1' )  THEN count <= count + "001";                                -- se incrementeaza cu 1 cand valid_p activ

END IF;

END IF;

END PROCESS;

--cat

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN

IF  ( reset = '1' )   THEN cat <= (others => '0');                    -- initializare cat

ELSIF (confirmare = '1')  THEN cat <= reg_a;                            -- atribuie catul care se afla in reg_a la starea de confirmare

END IF;

END IF;

END PROCESS;

--rest

PROCESS(clk)

BEGIN

IF (clk'EVENT AND clk = '1') THEN

IF  ( reset = '1' )   THEN rest <= (others => '0');                   -- initalizare rest

ELSIF (confirmare = '1')  THEN rest <= reg_p;                           -- i se atribuie restul din reg_p conform pdf-ului

END IF;

END IF;

END PROCESS;

pmsb <= reg_p(4);                                           -- bitul cel mai semnificativ al lui p

numarator <='1' WHEN count = "100" else '0';                -- numaratorul

END date;

Fișier control.vhd

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY control IS

PORT (

clk             : IN  std_logic;

reset           : IN  std_logic;

--FSM inputs

req             : IN  std_logic; --request

numarator       : IN  std_logic; -- cand se termina while 'numarator' <=1 altfel =0

pmsb            : IN  std_logic;

--FSM outputs

start             : OUT std_logic;

initizlizare      : OUT std_logic;

pauza             : OUT std_logic;

validare_p        : OUT std_logic;

adunare_b         : OUT std_logic;

adunare_minus_b   : OUT std_logic;

deplasare_a       : OUT std_logic;

validare_finala_p : OUT std_logic;

adunare           : OUT std_logic;

confirmare        : OUT std_logic;

ready             : OUT std_logic  --valid (BUSY)

);

END control;

ARCHITECTURE control OF control IS

TYPE state IS (S_START,S_INITIALIZARE,S_PAUZA,S_VALIDARE_P,S_ADUNARE_B,S_ADUNARE_MINUS_B,S_DEPLASARE_A,S_VALIDARE_FINALA_P,S_ADUNARE,S_CONFIRMARE);

SIGNAL currentState : state;

SIGNAL nextState    : state;

SIGNAL ld_r_ready   : std_logic;

BEGIN

CLC: PROCESS ( currentState, req, numarator, pmsb ) BEGIN

CASE currentState IS

WHEN S_START            => IF ( req   = '1' )     THEN

nextState <= S_INITIALIZARE;

ELSE

nextState <= S_START;

END IF;

WHEN S_INITIALIZARE     =>      nextState <= S_PAUZA;

WHEN S_PAUZA            => IF ( numarator = '1' ) THEN

nextState <= S_VALIDARE_FINALA_P;

ELSE

nextState <= S_VALIDARE_P;

END IF;

WHEN S_VALIDARE_P       => IF ( pmsb = '1' )      THEN

nextState <= S_ADUNARE_B;

ELSE

nextState <= S_ADUNARE_MINUS_B;

END IF;

WHEN S_ADUNARE_B        =>      nextState <= S_DEPLASARE_A;

WHEN S_ADUNARE_MINUS_B  =>      nextState <= S_DEPLASARE_A;

WHEN S_DEPLASARE_A      => IF ( numarator = '1' )  THEN

nextState <= S_VALIDARE_FINALA_P;

ELSE

nextState <= S_VALIDARE_P;

END IF;

WHEN S_VALIDARE_FINALA_P=> IF ( pmsb = '1' )       THEN

nextState <= S_ADUNARE;

ELSE

nextState <= S_CONFIRMARE;

END IF;

WHEN S_ADUNARE          =>      nextState <= S_CONFIRMARE;

WHEN S_CONFIRMARE       =>      nextState <= S_START;

END CASE;

END PROCESS CLC;

REG: PROCESS ( clk ) BEGIN

IF (clk'EVENT AND clk = '1') THEN

IF (reset = '1') THEN

currentState <= S_START;

ELSE

currentState <= nextState;

END IF;

END IF;

END PROCESS REG;

--FSM outputs

start             <= '1' WHEN (currentState = S_START             ) ELSE '0';

initizlizare      <= '1' WHEN (currentState = S_INITIALIZARE      ) ELSE '0';

pauza             <= '1' WHEN (currentState = S_PAUZA             ) ELSE '0';

validare_p        <= '1' WHEN (currentState = S_VALIDARE_P        ) ELSE '0';

adunare_b         <= '1' WHEN (currentState = S_ADUNARE_B         ) ELSE '0';

adunare_minus_b   <= '1' WHEN (currentState = S_ADUNARE_MINUS_B   ) ELSE '0';

deplasare_a       <= '1' WHEN (currentState = S_DEPLASARE_A       ) ELSE '0';

validare_finala_p <= '1' WHEN (currentState = S_VALIDARE_FINALA_P ) ELSE '0';

adunare           <= '1' WHEN (currentState = S_ADUNARE           ) ELSE '0';

confirmare        <= '1' WHEN (currentState = S_CONFIRMARE        ) ELSE '0';

ld_r_ready        <= '1' WHEN (currentState = S_START             ) ELSE '0';

-- ready is ld_r delayed

PROCESS ( clk ) BEGIN

IF (clk'EVENT AND clk = '1') THEN

IF (reset = '1') THEN

ready <= '0';

ELSE

ready <= ld_r_ready;

END IF;

END IF;

END PROCESS;

END control;

Fișier ifc.vhd

LIBRARY IEEE;

USE IEEE.std_logic_1164.ALL;

ENTITY ifc IS

PORT (

clk     : IN  std_logic;

reset   : IN  std_logic;

a       : IN  std_logic_vector(3 DOWNTO 0);

b       : IN  std_logic_vector(3 DOWNTO 0);

req     : IN  std_logic;

cat     : OUT std_logic_vector(3 DOWNTO 0);

rest    : OUT std_logic_vector(4 DOWNTO 0);

ready   : OUT  std_logic

);

END ifc;

ARCHITECTURE ifc OF ifc IS

COMPONENT date

PORT (

clk                 : IN  std_logic;

reset               : IN  std_logic;

a                   : IN  std_logic_vector(3 DOWNTO 0);

b                   : IN  std_logic_vector(3 DOWNTO 0);

start               : IN std_logic;

initizlizare        : IN std_logic;

pauza               : IN std_logic;

validare_p          : IN std_logic;

adunare_b           : IN std_logic;

adunare_minus_b     : IN std_logic;

deplasare_a         : IN std_logic;

validare_finala_p   : IN std_logic;

adunare             : IN std_logic;

confirmare          : IN std_logic;

cat                 : OUT std_logic_vector(3 DOWNTO 0);

rest                : OUT std_logic_vector(4 DOWNTO 0);

numarator           : OUT std_logic; --necesar pentru Calea de control

pmsb                : OUT std_logic --necesar pentru Calea de control

);

END COMPONENT;

COMPONENT control

PORT (

clk                 : IN  std_logic;

reset               : IN  std_logic;

--FSM inputs

req                 : IN  std_logic; --request

numarator           : IN  std_logic; -- cand se termina while 'numarator' <=1 altfel =0

pmsb                : IN  std_logic;

--FSM outputs

start               : OUT std_logic;

initizlizare        : OUT std_logic;

pauza               : OUT std_logic;

validare_p          : OUT std_logic;

adunare_b           : OUT std_logic;

adunare_minus_b     : OUT std_logic;

deplasare_a         : OUT std_logic;

validare_finala_p   : OUT std_logic;

adunare             : OUT std_logic;

confirmare          : OUT std_logic;

ready               : OUT std_logic  --valid (BUSY)

);

END COMPONENT;

-- signals to connect date/control

SIGNAL start,initizlizare,pauza,validare_p,adunare_b,adunare_minus_b,deplasare_a,validare_finala_p,adunare,confirmare,numarator,pmsb : std_logic;

BEGIN

i_date: date

PORT MAP(

clk                 => clk,

reset               => reset,

a                   => a,

b                   => b,

start               => start,

initizlizare        => initizlizare,

pauza               => pauza,

validare_p          => validare_p,

adunare_b           => adunare_b,

adunare_minus_b     => adunare_minus_b,

deplasare_a         => deplasare_a,

validare_finala_p   => validare_finala_p,

adunare             => adunare,

confirmare          => confirmare,

cat                 => cat,

rest                => rest,

numarator           => numarator,

pmsb                => pmsb

);

i_ctrl: control

PORT MAP(

clk                 => clk,

reset               => reset,

req                 => req,

numarator           => numarator,

pmsb                => pmsb,

start               => start,

initizlizare        => initizlizare,

pauza               => pauza,

validare_p          => validare_p,

adunare_b           => adunare_b,

adunare_minus_b     => adunare_minus_b,

deplasare_a         => deplasare_a,

validare_finala_p   => validare_finala_p,

adunare             => adunare,

confirmare          => confirmare,

ready               => ready

);

END ifc;

Fișier .c

#include <genlib.h>

main()

{

int i;

GENLIB_DEF_LOFIG("ifr_top");

GENLIB_LOCON("vdd",           IN,        "vdd");

GENLIB_LOCON("vss",           IN,        "vss");

GENLIB_LOCON("vdde",          IN,       "vdde");

GENLIB_LOCON("vsse",          IN,       "vsse");

GENLIB_LOCON("ck",            IN,       "ck");

GENLIB_LOCON("reset",         IN,      "reset");

GENLIB_LOCON("a[3:0]",        IN,     "a[3:0]");

GENLIB_LOCON("b[3:0]",        IN,     "b[3:0]");

GENLIB_LOCON("req",         IN,      "req");

GENLIB_LOCON("ready",        OUT,      "ready");

GENLIB_LOCON("cat[3:0]",    OUT,    "cat[3:0]");

GENLIB_LOCON("rest[4:0]",    OUT,  "rest[4:0]");

GENLIB_LOINS ("pvsse_sp", "p20", "cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS ("pvdde_sp", "p21", "cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS ("pvddeck_sp", "p22", "clock", "cki", "vdde", "vdd", "vsse", "vss",0);

GENLIB_LOINS ("pvssi_sp", "p23", "cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS ("pvddi_sp", "p24", "cki", "vdde", "vdd", "vsse", "vss", 0);

for (i = 0; i < 4; i++)

GENLIB_LOINS("pi_sp", GENLIB_NAME("p%d", i),

GENLIB_NAME("a[%d]", i), GENLIB_NAME("aa[%d]", i),

"cki", "vdde", "vdd", "vsse", "vss", 0);

for (i = 0; i < 4; i++)

GENLIB_LOINS("pi_sp", GENLIB_NAME("p%d", i + 4),

GENLIB_NAME("b[%d]", i), GENLIB_NAME("bb[%d]", i),

"cki", "vdde", "vdd", "vsse", "vss", 0);

for (i = 0; i < 4; i++)

GENLIB_LOINS("po_sp", GENLIB_NAME("p%d", i + 8),

GENLIB_NAME("catcat[%d]", i), GENLIB_NAME("cat[%d]", i),

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p25",

"restrest[0]", "rest[0]",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p12",

"restrest[1]", "rest[1]",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p13",

"restrest[2]", "rest[2]",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p14",

"restrest[3]", "rest[3]",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p15",

"restrest[4]", "rest[4]",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("pi_sp", "p16",

"req", "reqreq",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("pi_sp", "p17",

"reset", "resetreset",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("pck_sp", "p18",

"ck",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("po_sp", "p19",

"readyready", "ready",

"cki", "vdde", "vdd", "vsse", "vss", 0);

GENLIB_LOINS("ifr", "ifr","clock", "resetreset",

"aa[3:0]", "bb[3:0]","reqreq","readyready",

"catcat[3:0]","restrest[4:0]",

"vdd", "vss", 0);

GENLIB_SAVE_LOFIG();

exit(0);

}

Fișier .pat

in       a(3 DOWNTO 0);;;

in       b(3 DOWNTO 0);;;

in      clk;;;

in   reset;;;

in   req;;;

in     vdd;

in     vss;;;

out   ready;;;

out   cat(3 DOWNTO 0);;

out   rest(4 DOWNTO 0);;

BEGIN

-- Pattern description :

--         a    b    c   r   r   vv     r       c       r

--                   l   e   e   ds     e       a       e

--                   k   s   q   ds     a       t       s

--                       e              d               t

--                       t              y

pat_0   : 1111 0011  1   1   0   10     ?*    ?****  ?***** ;

pat_1   : 1111 0011  0   1   0   10     ?*    ?****  ?***** ;

pat_2   : 1111 0011  1   1   0   10     ?*    ?****  ?***** ;

pat_3   : 1111 0011  0   1   0   10     ?*    ?****  ?***** ;

pat_4   : 1111 0011  1   0   1   10     ?*    ?****  ?***** ;

pat_5   : 1111 0011  0   0   1   10     ?*    ?****  ?***** ;

pat_6   : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_7   : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_8   : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_9   : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_10  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_11  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_13  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_14  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_15  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_16  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_17  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_18  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_19  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_20  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_21  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_22  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_23  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_24  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_25  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_26  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_27  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_28  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_29  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_30  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_31  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_32  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_33  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_34  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

pat_35  : 1111 0011  1   0   0   10     ?*    ?****  ?***** ;

pat_36  : 1111 0011  0   0   0   10     ?*    ?****  ?***** ;

end;

Fișier .rin

#file: ifr.rin

width (vdd 100 vss 100)

west (p0 p1 p2 p3 p4 p5 p6 p7)

north (p16 p17 p18 p19 p25)

east (p8 p9 p10 p11 p12 p13 p14 p15)

south (p20 p21 p23 p24 p22)